Pon-Pon Alarcon: An Interactive Crocodile Dentist Board Game Using Buttons, Light, and Proximity Sensors

- System overview

The Pon-Pon Alarcon project is inspired by the classic “Crocodile Dentist” toy, where players take turns pressing the crocodile’s teeth not knowing which one will trigger the mouth to snap. Growing up this kind of game is simple but surprisingly exciting because of the suspense it creates. With that idea the group aimed to recreate and enhance the experience by integrating electronics and sensor-based interactions. Instead of relying purely on mechanical triggers the project introduces a more dynamic and unpredictable gameplay using sensors and programmable logic. This makes the game more interactive and slightly more challenging since outcomes are not fixed and can change depending on different conditions like light and distance. From a practical perspective the project also serves as a hands-on application of embedded systems. It combines both hardware and software concepts that are commonly discussed in class but not always experienced in a real working prototype. Through this the group was able to better understand how different components communicate and work together in a single system.

Objectives of the Project

- To develop an interactive electronic board game  the goal is to transform a traditional toy into a more engaging system using sensors and programmable components, making gameplay less predictable and more exciting.

- To integrate multiple sensors into a single system by using a photoresistor, ultrasonic sensor, and potentiometer, the project demonstrates how different types of inputs can influence one application.

- To provide real-time feedback using output the system uses a servo motor, buzzer, LCD, and RGB LED to give immediate responses, helping players understand what is happening during the game.

- To simulate randomness and dynamic behavior instead of a fixed trigger, the system uses sensor readings and programmed logic to decide outcomes, making each game session different.

- To enhance understanding of hardware-software interaction the project helps in visualizing how code directly affects physical components, which is important in embedded systems development.

- Description of sensors, outputs, and modules used

The project uses a combination of sensors, input devices, and output components that each play a specific role in the gameplay.

Sensors

- The photoresistor (LDR) is used to detect the level of light in the environment. Instead of just measuring brightness for display, it actually affects the behavior of the game. For example, in brighter conditions, the system may become more sensitive or increase the chances of triggering the crocodile. This adds an environmental factor that players might not immediately notice but still affects the outcome.

- The ultrasonic sensor is responsible for detecting the distance between the player’s hand and the device. It was included to make the system feel more interactive, rather than just relying on button presses. When a player’s hand is too close, it can influence the system’s decision-making, almost like the crocodile is “reacting” to movement.

- The potentiometer acts as a manual control for adjusting certain parameters of the game. Instead of hardcoding everything, this allows flexibility, such as changing the difficulty level or sensitivity. It’s a simple component, but it gives more control during testing and gameplay.

Inputs

The system includes six push buttons each representing a crocodile tooth. These are the main interaction points for the player. Every press is treated as a decision point in the game. Instead of assigning a fixed “losing button,” the system evaluates conditions every time a button is pressed, which makes the experience less predictable.

Outputs

- The servo motor is one of the most noticeable parts of the system. It controls the snapping motion of the crocodile’s mouth. When triggered, it quickly moves to simulate the bite, which gives a physical and visual reaction.

- The passive buzzer adds sound effects to the game. It plays different tones depending on the situation, such as a warning sound or a “game over” signal. This makes the experience more immersive, even if the sound is simple.

- The 16x2 LCD display is used to show messages like instructions, game status, or results. Instead of leaving the player guessing, it provides clear feedback on what is happening in the system.

- The RGB LED is used for visual indicators. Different colors represent different states for example, green for safe, red for danger, and possibly blue for standby. It helps communicate the system’s status quickly without needing to read the LCD all the time.

- Application flow and logic explanation

- The gameplay starts once the system is powered on. The LCD initializes and displays a message prompting the player to begin. At this point, the RGB LED may show a neutral color indicating that the system is ready.

- When a player presses one of the buttons, the system does not immediately decide the result. Instead, it first reads input values from the sensors. The light level from the photoresistor, the distance detected by the ultrasonic sensor, and the value from the potentiometer are all considered.

- These inputs are then processed in the program. Rather than using a fixed condition, the system combines these values with a form of programmed randomness. This means that even if the same button is pressed multiple times, the result can still be different depending on the environment and settings.

- If the computed result is safe, the game continues. The LED might briefly show green, and the LCD updates to encourage the player to keep going. However, if the conditions meet the trigger threshold, the system activates the losing sequence.

- In that moment, the servo motor quickly moves to simulate the crocodile snapping its mouth. At the same time, the buzzer produces a sound effect, and the RGB LED changes to red. The LCD displays a “Game Over” message, clearly indicating the result.

- One thing noticeable during testing is that the game does not feel repetitive because of how the inputs affect the outcome. Even small changes, like lighting or hand distance, can slightly alter the behavior, which adds to the unpredictability.

- Hardware–software interaction overview

The entire system is controlled by a microcontroller (such as an Arduino), which acts as the central unit connecting all components. Each sensor and output device is wired to specific pins, allowing the program to read inputs and send commands. When a player presses a button, a signal is sent to the microcontroller. The program then reads the current values from the sensors almost immediately. These values are processed using conditional statements and logic written in the code. For example when the button is pressed the system might check:

- Is the light level above a certain threshold?

- Is the player’s hand too close?

- What is the current potentiometer setting?

Based on these conditions, the program decides whether to continue the game or trigger the crocodile. If the result is safe, the microcontroller sends signals to the RGB LED and LCD to display the appropriate feedback. If not, it sends a command to the servo motor to rotate quickly, while also activating the buzzer and updating the display. A simple interaction would look like this When the player presses a button, the system pauses for a short moment, reads all sensor data, and then suddenly either does nothing (safe) or reacts with movement, sound, and light (triggered). That short delay actually adds tension to the game, which was not originally planned but turned out to improve the experience.

e. Screenshots or photos of the working system